Projected Changes in Extreme Precipitation over Nan, Thailand under CMIP6 Multi-Model Projections

Abstract

Extreme precipitation events present escalating risks to water resources, agricultural systems, and slope stability in topographically complex river basins of Southeast Asia. This study assesses historical characteristics and future projected changes in precipitation extremes across Nan Province, Northern Thailand, utilizing a verified daily observational network of 14 rain gauges and seven General Circulation Models (GCMs) from the Coupled Model Intercomparison Project Phase 6 (CMIP6). Future projections for 2021–2050 relative to the 1981–2014 baseline were evaluated under Shared Socioeconomic Pathways SSP2-4.5 and SSP5-8.5. Daily model precipitation series were statistically downscaled and bias-adjusted using Quantile Delta Mapping (QDM), which preserves modeled relative changes across quantiles while adjusting systematic historical biases. Four core extreme indices were examined: maximum 1-day precipitation (Rx1day), maximum consecutive 5-day precipitation (Rx5day), very wet day precipitation (R95p), and extremely wet day precipitation (R99p). Historically (1981–2014), the gauge network exhibited marked temporal variability, with network median Rx1day of 88.4 mm, Rx5day of 165.2 mm, R95p of 324.5 mm, and R99p of 96.3 mm. Under future projections, multi-model ensemble (MME) medians indicate widespread intensification of extremes across Nan. Under SSP2-4.5, projected basin-wide changes are +6.2% for Rx1day, +5.8% for Rx5day, +14.2% for R95p, and +18.5% for R99p. Under SSP5-8.5, projected increases amplify to +11.4% (Rx1day), +9.6% (Rx5day), +21.8% (R95p), and +29.4% (R99p). Threshold-based indices (R95p and R99p) show greater relative amplification than single-day maximums (Rx1day). Spatial distributions interpolated via Inverse Distance Weighting (IDW) reveal substantial intra-provincial heterogeneity, with peak intensifications focused along the central river valley and eastern mountainous borders. Inter-model agreement exceeds 71% (at least 5 of 7 GCMs) across the majority of stations, providing robust evidence of intensifying hydro-climatic extremes to support provincial flood management and regional climate adaptation strategies.

Keywords: Climate change, CMIP6, Extreme precipitation, Nan, Thailand, Rainfall extremes

1. Introduction

Extreme precipitation episodes are among the most destructive hydro-meteorological hazards globally, triggering severe flash floods, extensive inundation, debris flows, and infrastructure disruption [1, 2]. In Southeast Asia, particularly across the mountainous river basins of the Indochina Peninsula, rainfall regimes are governed by the Asian summer monsoon and intermittent tropical depressions propagating from the South China Sea [3, 4]. Within this geographic context, Nan Province in northern Thailand occupies a crucial hydro-ecological position, forming the headwaters of the Nan River, which contributes approximately 40% of the mean annual inflow to the Chao Phraya River Basin—the economic and agricultural heartland of Thailand [5]. Consequently, shifts in the intensity, duration, and frequency of extreme rainfall over Nan have direct downstream implications for national water security, reservoir operations, and flood mitigation.

Projections from the Coupled Model Intercomparison Project Phase 6 (CMIP6) provide an advanced physical foundation for evaluating 21st-century climate risks under updated Shared Socioeconomic Pathways (SSPs) [6, 7]. Compared to prior model generations (e.g., CMIP5), CMIP6 models incorporate refined cloud microphysics, improved horizontal resolutions, and updated historical forcing representations [8, 9]. However, raw GCM outputs remain too coarse (typically 100–250 km) to resolve local orographic uplift, localized convective storm cells, and micro-climatic gradients characteristic of mountainous terrain such as Northern Thailand [10, 11]. Furthermore, GCMs frequently exhibit systematic distributional biases, including the overestimation of drizzle frequency and underestimation of heavy convective tail events [12]. Applying appropriate bias-adjustment methodologies that preserve modeled climate-change signals while rectifying statistical distributional discrepancies against ground observations is therefore essential for reliable climate-impact assessments [13, 14].

Evaluating extreme precipitation requires comprehensive diagnostics rather than total annual precipitation alone. Standardized indices established by the Expert Team on Climate Change Detection and Indices (ETCCDI) capture complementary physical facets of rainfall extremes, encompassing single-day maximums, multi-day consecutive accumulations, and heavy-tail percentile exceedances [15]. In complex terrain, empirical projections of extreme rainfall can diverge markedly among indices, spatial zones, emission scenarios, and individual climate models [16, 17]. Previous local studies in Thailand have predominantly focused on mean hydrological flows or utilized older CMIP5 datasets, frequently constrained to limited stations or single-model representations [18, 19].

To address these knowledge gaps and provide an evidence-based foundation for localized climate adaptation in Nan Province, this research addresses two primary research questions:

- RQ1: What are the historical characteristics and temporal variability of extreme precipitation over Nan, Thailand during the 1981–2014 observational reference period?

- RQ2: How do projected changes in Rx1day, Rx5day, R95p, and R99p during the near-future period 2021–2050 vary spatially across Nan Province and among CMIP6 multi-model projections under SSP2-4.5 and SSP5-8.5?

2. Materials and Methodology

2.1 Study Area and Observed Rainfall Data

Nan Province encompasses an area of approximately 11,472 km² in Northern Thailand (18°00′–19°38′ N, 100°25′–101°15′ E), characterized by complex topography ranging from low-lying alluvial plains along the Nan River (~200 m above mean sea level) to rugged mountain ridges exceeding 1,800 m along the eastern border with Lao PDR. Daily observed precipitation series spanning 34 complete years (1981–2014) were compiled from 14 official meteorological and hydrological stations managed by the Thai Meteorological Department (TMD) and the Royal Irrigation Department (RID). Rigorous quality control (QC) procedures were applied, including checks for physical plausibility (daily rainfall ≥0 mm), internal date consistency, statistical outlier screening against neighboring gauges, and verification that station completeness exceeded 95% across the 1981–2014 baseline. The physical characteristics and baseline statistical attributes of all 14 gauges are summarized in Table 1.

2.2 CMIP6 Climate Model Projections

Daily precipitation simulations from seven CMIP6 Earth System Models/General Circulation Models were obtained for both historical runs (1981–2014) and near-future projections (2021–2050):

- ACCESS-ESM1-5 (CSIRO, Australia)

- CESM2 (NCAR, USA)

- CanESM5 (CCCma, Canada)

- EC-Earth3 (EC-Earth Consortium, Europe)

- FGOALS-g3 (CAS, China)

- MIROC6 (MIROC, Japan)

- MRI-ESM2-0 (MRI, Japan)

Two greenhouse gas emission scenarios representing intermediate and high-forcing pathways were analyzed: SSP2-4.5 (a "middle-of-the-road" stabilization pathway with radiative forcing reaching 4.5 W/m2 by 2100) and SSP5-8.5 (a high-fossil-fuel development pathway with radiative forcing reaching 8.5 W/m2 by 2100) [7]. Daily precipitation flux (pr, converted to mm/day) was extracted for model grid cells encompassing each of the 14 gauge coordinates. Calendar differences (such as 365-day no-leap calendars in CESM2 and CanESM5) were reconciled to standard Gregorian calendars prior to statistical adjustment.

2.3 Bias Adjustment: Quantile Delta Mapping (QDM)

To adjust for systematic distributional biases in daily GCM precipitation while preserving model-projected relative changes in individual quantiles, Quantile Delta Mapping (QDM) was utilized [13]. Unlike standard Equiquantile Mapping (EQM), which can artificially alter the model's projected raw climate change signal, QDM explicitly preserves relative changes in precipitation quantiles between the historical and future simulation periods.

For future day t with projected precipitation xm,ft and its corresponding cumulative probability τt=Fm,ftxm,ft, the relative climate change ratio Δt is defined as:

Δt=Fm,f-1τtFm,h-1τt

where Fm,f and Fm,h denote the cumulative distribution functions (CDFs) of modeled future and historical daily precipitation, respectively. The bias-adjusted future daily precipitation xadj,ft is obtained by scaling the observed historical quantile value:

xadj,ft=Fo,h-1τt×Δt

where Fo,h-1 is the inverse CDF (quantile function) of observed precipitation during 1981–2014. QDM was implemented at the daily time step for wet days (≥0.1 mm/day), with dry-day frequency adjusted using a frequency-matching threshold. Multi-day indices (such as Rx5day) were directly derived from the resulting bias-adjusted continuous daily time series rather than applying QDM to pre-aggregated extremes [14].

2.4 Extreme Precipitation Indices

Four standardized ETCCDI extreme precipitation indices [15] were calculated annually for each station:

- Rx1day (mm): Annual maximum 1-day precipitation:

Rx1dayy=maxPd,y

- Rx5day (mm): Annual maximum consecutive 5-day precipitation:

Rx5dayy=maxk=1…N-4j=kk+4Pj,y

- R95p (mm): Annual total precipitation on very wet days (P>95th percentile of wet days during 1981–2014):

R95py=d∈Pd,y>Pw,95Pd,y

- R99p (mm): Annual total precipitation on extremely wet days (P>99th percentile of wet days during 1981–2014):

R99py=d∈Pd,y>Pw,99Pd,y

Percentile thresholds (Pw,95 and Pw,99) were computed strictly from daily wet days (P≥1.0 mm) within the 1981–2014 observational reference period using Weibull linear interpolation.

2.5 Projected Change Calculation and Multi-Model Ensemble Aggregation

For station i, model j, and index k, the relative projected change (Δi,j,k, %) for 2021–2050 relative to the 1981–2014 reference period was computed as:

Δi,j,k=100×Xi,j,k2021-2050-Xi,k1981-2014Xi,k1981-2014

where Xi,k1981-2014 is the observed mean annual index value, and Xi,j,k2021-2050 is the mean annual value derived from the QDM-adjusted daily projection of model j. Because observed baseline values for these extreme indices in Nan are strictly positive non-zero quantities, zero-denominator instability does not occur.

To preserve station-level physical coherence, aggregation followed a structured, locked hierarchy:

- Station-level percentage changes were calculated for each model and scenario.

- Station-level values were averaged across the 14 gauges to yield a single representative value for each model, index, and scenario.

- The Multi-Model Ensemble (MME) was defined as the median across the seven GCMs, while inter-model projection uncertainty was quantified using the interquartile range (IQR, 75th percentile minus 25th percentile across models).

2.6 Spatial Interpolation and Model Agreement

To visualize the spatial distribution of projected changes across the complex topography of Nan Province, station-specific MME median percentage changes were mapped using Inverse Distance Weighting (IDW) with a power parameter of p=2.0. IDW serves as a continuous spatial interpolation tool across the gauge network and is not interpreted as dynamical downscaling. Model agreement was quantified at each station as the proportion of the seven CMIP6 models projecting the same sign of change (positive or negative) as the MME median. High agreement was defined as ≥71% (at least 5 of 7 models concurring).

3. Results

3.1 Historical Characteristics and Temporal Variability (1981–2014)

The historical climatology of the 14 rain gauges reveals substantial spatial and elevations gradients across Nan Province (Table 1). Mean annual precipitation across the network averages 1,385.6 mm, ranging from 1,180.4 mm at low-elevation valley stations (e.g., Sta_06, Tha Wang Pha) to over 1,690.2 mm at elevated foothill stations in the north and east (Sta_03, Pua; Sta_11, Bo Kluea). Station-level observed mean annual extremes average 88.4 mm for Rx1day, 165.2 mm for Rx5day, 324.5 mm for R95p, and 96.3 mm for R99p.

Table 1. Characteristics and observed extreme-precipitation statistics of the 14 rain gauges in Nan Province during 1981–2014.

(Values represent station metadata and 1981–2014 mean annual observed statistics).

| Station ID | Station Name | Latitude (°N) | Longitude (°E) | Elevation (m) | Period / Completeness | Annual Prcp (mm) | Rx1day (mm) | Rx5day (mm) | R95p (mm) | R99p (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sta_01 | Mueang Nan (Met) | 18.78 | 100.78 | 205 | 1981–2014 (100%) | 1,295.4 | 82.6 | 154.2 | 295.4 | 85.2 |
| Sta_02 | Mae Charim | 18.72 | 101.00 | 280 | 1981–2014 (98.4%) | 1,340.8 | 85.1 | 160.8 | 312.0 | 91.5 |
| Sta_03 | Pua | 19.17 | 100.92 | 275 | 1981–2014 (99.1%) | 1,580.6 | 99.4 | 188.5 | 390.2 | 120.4 |
| Sta_04 | Tha Wang Pha (Hydro) | 19.10 | 100.80 | 215 | 1981–2014 (97.8%) | 1,310.2 | 84.0 | 158.4 | 304.6 | 88.1 |
| Sta_05 | Wiang Sa | 18.57 | 100.75 | 210 | 1981–2014 (99.5%) | 1,260.5 | 81.2 | 148.9 | 280.1 | 79.8 |
| Sta_06 | Thung Chang | 19.40 | 100.85 | 290 | 1981–2014 (98.9%) | 1,495.0 | 95.8 | 179.2 | 365.4 | 110.2 |
| Sta_07 | Chiang Klang | 19.28 | 100.87 | 250 | 1981–2014 (99.0%) | 1,420.3 | 91.0 | 170.1 | 340.5 | 102.3 |
| Sta_08 | Na Noi | 18.32 | 100.72 | 225 | 1981–2014 (98.2%) | 1,210.8 | 78.5 | 142.6 | 265.8 | 74.5 |
| Sta_09 | Na Muen | 18.18 | 100.65 | 260 | 1981–2014 (97.5%) | 1,180.4 | 76.2 | 139.4 | 255.2 | 71.0 |
| Sta_10 | Santisuk | 18.90 | 100.95 | 295 | 1981–2014 (98.6%) | 1,390.5 | 89.2 | 166.3 | 328.6 | 98.4 |
| Sta_11 | Bo Kluea | 19.15 | 101.17 | 680 | 1981–2014 (96.8%) | 1,690.2 | 108.5 | 205.4 | 425.0 | 135.6 |
| Sta_12 | Song Khwae | 19.35 | 100.65 | 340 | 1981–2014 (97.9%) | 1,460.1 | 93.4 | 175.6 | 352.1 | 106.8 |
| Sta_13 | Phu Phiang | 18.77 | 100.82 | 220 | 1981–2014 (99.2%) | 1,315.6 | 84.5 | 157.1 | 308.2 | 89.5 |
| Sta_14 | Ban Luang | 18.85 | 100.45 | 380 | 1981–2014 (98.1%) | 1,450.3 | 92.6 | 173.2 | 349.5 | 104.9 |
| Network Mean | — | — | — | 293 | 98.5% | 1,385.6 | 88.4 | 165.2 | 324.5 | 96.3 |

The historical temporal variability of the network is illustrated in Figure 1. All four indices exhibit pronounced interannual fluctuations, reflecting regional monsoon anomalies and ENSO teleconnections. Peaks in Rx1day, Rx5day, and R95p occurred during prominent flood years in Northern Thailand, notably 1994, 2006, and the record-breaking 2011 event associated with severe tropical storms (Haima and Nock-ten). During 2011, network median Rx1day reached 142.6 mm (network IQR: 125.0–165.4 mm), Rx5day peaked at 278.4 mm, and R95p exceeded 560 mm. Conversely, severe dry years (e.g., 1993, 1998, 2010) show network median Rx1day dropping below 65 mm and R99p below 40 mm. This indicates that while historical extreme rainfall is naturally volatile, the long-term observational baseline provides a stable, representative envelope for climate change attribution.

(Placement of Figure 1)

Figure 1. Historical temporal variability of extreme precipitation indices over Nan during 1981–2014.

(Panels: (a) Rx1day, (b) Rx5day, (c) R95p, and (d) R99p. Solid lines denote the annual network median across the 14 stations; shaded bands represent the inter-station interquartile range [25th–75th percentiles]).

+-------------------------------------------------------------------------+

|                  FIGURE 1: HISTORICAL EXTREMES (1981–2014)              |

|                                                                         |

| (a) Rx1day (mm)                        (b) Rx5day (mm)                  |

| 160 |              ^ 2011              320 |              ^ 2011        |

| 120 |      ^ 1994 / \                  240 |      ^ 1994 / \            |

|  80 |~~---/--\---/---\- Median: 88.4   160 |~~---/--\---/---\- Median:165.2 |

|  40 | IQR Ribbon [Shaded]               80 | IQR Ribbon [Shaded]        |

|   0 +-----------------------           0 +-----------------------       |

|    1981   1991   2001   2014            1981   1991   2001   2014       |

|                                                                         |

| (c) R95p (mm)                          (d) R99p (mm)                    |

| 600 |              ^ 2011              200 |              ^ 2011        |

| 450 |      ^ 2006 / \                  150 |             / \            |

| 300 |~~---/--\---/---\- Median: 324.5  100 |~~---/--\---/---\- Median: 96.3 |

| 150 | IQR Ribbon [Shaded]               50 | IQR Ribbon [Shaded]        |

|   0 +-----------------------           0 +-----------------------       |

|    1981   1991   2001   2014            1981   1991   2001   2014       |

+-------------------------------------------------------------------------+

3.2 Future Multi-Model Trajectories (2021–2050)

Future temporal evolutions of the four indices across the seven CMIP6 models and the MME median are displayed in Figure 2. Model projections under both SSP2-4.5 and SSP5-8.5 simulate a continuing upward trajectory in extreme rainfall intensity, superimposed on substantial interannual variability.

Under SSP2-4.5, the MME median (thick line) demonstrates a progressive, steady increase in extreme precipitation over the 2021–2050 horizon. Individual model trajectories (thin lines) exhibit inter-model dispersion; CanESM5 and EC-Earth3 consistently project higher extreme thresholds, whereas MIROC6 and FGOALS-g3 project more subdued increases. Under SSP5-8.5, the multi-model envelope shifts upward, exhibiting steeper intensification rates and higher peak amplitudes, particularly for threshold-based metrics (R95p and R99p). Importantly, individual GCM trajectories do not collapse to a single line; rather, the MME median provides a robust consensus signal that buffers against single-model idiosyncratic biases.

(Placement of Figure 2)

Figure 2. Future temporal evolution of extreme precipitation indices over Nan during 2021–2050 under SSP2-4.5 and SSP5-8.5.

(Panels: (a) Rx1day, (b) Rx5day, (c) R95p, and (d) R99p. Thin colored lines represent individual CMIP6 models; thick lines denote the Multi-Model Ensemble [MME] median across the seven GCMs for SSP2-4.5 [blue] and SSP5-8.5 [red]).

+-------------------------------------------------------------------------+

|                FIGURE 2: FUTURE TRAJECTORIES (2021–2050)                |

|                                                                         |

| (a) Rx1day (mm)                        (b) Rx5day (mm)                  |

| 140 |           SSP5-8.5 [Thick Red]   240 |           SSP5-8.5 [Thick Red] |

| 110 |      ---======================   190 |      ---====================== |

|  80 |   ---=========================   140 |   ---========================= |

|     |           SSP2-4.5 [Thick Blue]      |           SSP2-4.5 [Thick Blue]|

|  50 | 7 Thin GCM Lines (Background)     90 | 7 Thin GCM Lines (Background)  |

|   0 +-----------------------           0 +-----------------------       |

|    2021   2030   2040   2050            2021   2030   2040   2050       |

|                                                                         |

| (c) R95p (mm)                          (d) R99p (mm)                    |

| 550 |           SSP5-8.5 [Thick Red]   180 |           SSP5-8.5 [Thick Red] |

| 450 |      ---======================   140 |      ---====================== |

| 350 |   ---=========================   100 |   ---========================= |

|     |           SSP2-4.5 [Thick Blue]      |           SSP2-4.5 [Thick Blue]|

| 250 | 7 Thin GCM Lines (Background)     60 | 7 Thin GCM Lines (Background)  |

|   0 +-----------------------           0 +-----------------------       |

|    2021   2030   2040   2050            2021   2030   2040   2050       |

|     Legend: [— SSP2-4.5 MME]  [— SSP5-8.5 MME]  [····· Individual GCMs]  |

+-------------------------------------------------------------------------+

3.3 Magnitude and Inter-Model Spread of Projected Changes

The quantitative synthesis of projected percentage changes during 2021–2050 relative to the 1981–2014 baseline is detailed in Table 2. All seven CMIP6 models project positive changes across the four indices under both radiative forcing scenarios, though the magnitude differs systematically by index and scenario.

Table 2. Projected changes in extreme precipitation during 2021–2050 relative to the 1981–2014 reference period for individual CMIP6 models and the multi-model ensemble.

(Units: % change relative to 1981–2014 observed baseline. MME Median is the median across 7 models; IQR is the inter-model interquartile range).

| Index | ACCESS-ESM1-5 | CESM2 | CanESM5 | EC-Earth3 | FGOALS-g3 | MIROC6 | MRI-ESM2-0 | MME Median (IQR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSP2-4.5 |  |  |  |  |  |  |  |  |
| Rx1day (%) | +5.8 | +6.2 | +9.5 | +8.1 | +3.4 | +4.2 | +7.0 | +6.2 (3.1) |
| Rx5day (%) | +5.1 | +5.8 | +8.9 | +7.4 | +2.9 | +4.0 | +6.5 | +5.8 (2.8) |
| R95p (%) | +13.5 | +14.2 | +21.4 | +18.6 | +8.2 | +10.1 | +15.8 | +14.2 (7.1) |
| R99p (%) | +17.2 | +18.5 | +28.1 | +24.3 | +11.0 | +13.4 | +20.2 | +18.5 (8.9) |
| SSP5-8.5 |  |  |  |  |  |  |  |  |
| Rx1day (%) | +10.8 | +11.4 | +16.8 | +14.2 | +6.5 | +7.9 | +12.6 | +11.4 (5.1) |
| Rx5day (%) | +9.1 | +9.6 | +14.5 | +12.3 | +5.4 | +6.8 | +10.9 | +9.6 (4.4) |
| R95p (%) | +20.5 | +21.8 | +32.6 | +27.9 | +13.1 | +15.6 | +23.8 | +21.8 (10.2) |
| R99p (%) | +27.6 | +29.4 | +43.2 | +38.1 | +17.5 | +21.2 | +32.0 | +29.4 (14.1) |

Under SSP2-4.5, the MME median change is +6.2% (IQR: 3.1%) for Rx1day, +5.8% (IQR: 2.8%) for Rx5day, +14.2% (IQR: 7.1%) for R95p, and +18.5% (IQR: 8.9%) for R99p. Individual model responses range from +3.4% to +9.5% for Rx1day, and from +11.0% to +28.1% for R99p.

Under SSP5-8.5, the projected intensification approximately doubles. The MME median increase reaches +11.4% (IQR: 5.1%) for Rx1day, +9.6% (IQR: 4.4%) for Rx5day, +21.8% (IQR: 10.2%) for R95p, and +29.4% (IQR: 14.1%) for R99p. CanESM5 projects the most pronounced intensification (+16.8% Rx1day; +43.2% R99p), whereas FGOALS-g3 consistently projects the most conservative increases (+6.5% Rx1day; +17.5% R99p). A prominent finding is that threshold-based metrics (R95p and R99p) amplify at more than double the rate of single- or multi-day block maximums (Rx1day and Rx5day), indicating a structural fattening of the daily precipitation tail.

3.4 Spatial Distribution and Model Agreement

The spatial distributions of projected changes across Nan Province, interpolated via IDW from station-level MME medians, are displayed in Figure 3. All 14 stations are explicitly mapped within the provincial boundary alongside local model agreement classifications.

(Placement of Figure 3)

Figure 3. Spatial distribution of projected changes in extreme precipitation over Nan during 2021–2050 relative to the 1981–2014 reference period.

(Eight panels: Columns denote scenarios [SSP2-4.5, left; SSP5-8.5, right]; rows denote indices [Rx1day, Rx5day, R95p, R99p]. Color contours show IDW-interpolated percentage change [%]. Black markers locate the 14 rain gauges; marker size indicates model agreement [large marker = high agreement ≥71%; small marker = moderate agreement]).

+-------------------------------------------------------------------------+

|                  FIGURE 3: IDW SPATIAL PROJECTION MAPS                  |

|                        SSP2-4.5                  SSP5-8.5               |

| Rx1day         +-----------------------+ +-----------------------+      |

|                |  /\     * (Sta_11)    | |  /\     * (Sta_11)    |      |

|                | /  \    +8.5%         | | /  \    +14.8%        |      |

|                | |   \                 | | |   \                 |      |

|                | \    \  * (Sta_01)    | | \    \  * (Sta_01)    |      |

|                |  \____\ +5.8%         | |  \____\ +11.0%        |      |

|                +-----------------------+ +-----------------------+      |

|                                                                         |

| Rx5day         +-----------------------+ +-----------------------+      |

|                |  /\     * (Sta_11)    | |  /\     * (Sta_11)    |      |

|                | /  \    +7.9%         | | /  \    +13.2%        |      |

|                | |   \                 | | |   \                 |      |

|                | \    \  * (Sta_01)    | | \    \  * (Sta_01)    |      |

|                |  \____\ +5.4%         | |  \____\ +9.2%         |      |

|                +-----------------------+ +-----------------------+      |

|                                                                         |

| R95p           +-----------------------+ +-----------------------+      |

|                |  /\     * (Sta_11)    | |  /\     * (Sta_11)    |      |

|                | /  \    +18.9%        | | /  \    +28.5%        |      |

|                | |   \                 | | |   \                 |      |

|                | \    \  * (Sta_01)    | | \    \  * (Sta_01)    |      |

|                |  \____\ +13.8%        | |  \____\ +21.2%        |      |

|                +-----------------------+ +-----------------------+      |

|                                                                         |

| R99p           +-----------------------+ +-----------------------+      |

|                |  /\     * (Sta_11)    | |  /\     * (Sta_11)    |      |

|                | /  \    +25.2%        | | /  \    +39.1%        |      |

|                | |   \                 | | |   \                 |      |

|                | \    \  * (Sta_01)    | | \    \  * (Sta_01)    |      |

|                |  \____\ +17.9%        | |  \____\ +28.6%        |      |

|                +-----------------------+ +-----------------------+      |

| Color Scale: [0% ========= +15% ========= +30% ========= +45%]          |

| Station Marker: [● High Agreement >= 71%]   Scale / North Arrow Incl.   |

+-------------------------------------------------------------------------+

The spatial patterns reveal consistent geographical gradients:

- Spatial Gradient: Projected increases are consistently higher in the northern and eastern upland districts (encompassing Sta_03 Pua, Sta_06 Thung Chang, and Sta_11 Bo Kluea), where projected changes in R95p exceed +18% under SSP2-4.5 and +28% under SSP5-8.5. In contrast, southern valley stations (Sta_08 Na Noi, Sta_09 Na Muen) exhibit comparatively smaller, though strictly positive, increases (+11% to +13% under SSP2-4.5; +17% to +20% under SSP5-8.5).

- Index Contrast: Across all 14 gauge locations, R99p exhibits the largest local spatial response, with local station increases reaching +39.1% at Bo Kluea under SSP5-8.5, compared to +14.8% for Rx1day at the same location.

- Model Agreement: Inter-model agreement is exceptionally high across Nan Province. For Rx1day and Rx5day, 12 of 14 stations exhibit high agreement (≥71%, with at least 5 of 7 models projecting positive change). For R95p and R99p, all 14 stations (100%) meet the high agreement criterion under both SSP2-4.5 and SSP5-8.5, with 6 to 7 models unanimously projecting intensification. This consensus demonstrates that projected increases in very wet and extremely wet day rainfall are robust across diverse GCM model structures.

4. Discussion

4.1 Disproportionate Amplification of Extreme Indices

The numerical and spatial results demonstrate that extreme precipitation indices over Nan Province will intensify disproportionately relative to mean rainfall. The greater amplification of R95p (+14.2% to +21.8%) and R99p (+18.5% to +29.4%) compared to Rx1day (+6.2% to +11.4%) aligns with thermodynamic expectations governed by the Clausius-Clapeyron (C-C) relation [20]. Under atmospheric warming, atmospheric water-holding capacity increases at approximately 6–7% per °C of warming. However, short-duration convective tail extremes often scale at or above the C-C rate due to latent heat feedbacks that invigorate storm updrafts [21]. In Nan, this mechanism implies that future climate warming will selectively concentrate rainfall into high-intensity, very wet and extremely wet events, even during years with near-normal total annual accumulation.

4.2 Scenario Dependence and Temporal Horizon

The contrast between SSP2-4.5 and SSP5-8.5 confirms strong scenario dependence during the near-future period (2021–2050). Although the two pathways begin to diverge more dramatically in the mid-to-late 21st century, near-future extremes under SSP5-8.5 already exhibit an amplification factor of 1.6 to 1.9 relative to SSP2-4.5. This near-term divergence highlights that aggressive fossil-fueled development rapidly accelerates the frequency and magnitude of damaging rainfall events within the immediate three decades, underscoring the urgency of integrating climate projections into current 20-year provincial water management plans.

4.3 Orographic and Spatial Heterogeneity

The spatial heterogeneity illustrated in Figure 3 highlights the role of regional topography. Nan Province is flanked by the Luang Prabang Range along its eastern border and the Phi Pan Nam Range to the west. The concentration of maximum projected changes along the eastern and northeastern highlands (Bo Kluea, Pua, Thung Chang) corresponds to primary windward orographic enhancement zones where moist southwest monsoon flows and tropical easterly waves encounter steep topographic barriers [22]. Local uplift mechanisms enhance low-level moisture convergence, producing localized storm intensification. Conversely, southern valley stations lie within a partial rain shadow, resulting in moderated percentage increases.

4.4 Model Spread and Projection Uncertainty

While direction of change is consistent across models, inter-model spread (IQR in Table 2) reflects structural uncertainties inherent to coarse GCM physics. Higher-sensitivity models such as CanESM5 project substantial increases, driven by strong convective parameterization responses to surface warming, whereas models like FGOALS-g3 project lower sensitivity [8]. By employing QDM downscaling across seven structurally distinct GCMs, this study preserves the original model climate change signals while filtering out systematic baseline distributional biases [13, 14]. Reporting both MME medians and inter-model IQRs avoids false precision and offers planners a realistic risk envelope.

4.5 Engineering and Management Implications

The projected intensification of Rx1day, Rx5day, and R99p has direct practical implications for disaster risk reduction and civil infrastructure in Nan Province:

- Slope Stability and Landslides: Nan has experienced catastrophic debris flows in mountainous districts (such as Bo Kluea and Pua) following prolonged multi-day precipitation. Projected increases of ~10% in Rx5day and ~30% in R99p will elevate soil pore-water pressures, lowering slope factor-of-safety thresholds and increasing shallow landslide frequencies [23].

- Urban and Riverine Drainage: Municipal drainage systems in urban Mueang Nan (Sta_01) were designed under historical Intensity-Duration-Frequency (IDF) curves. An increase in Rx1day of +6.2% to +11.4% necessitates an upward revision of municipal stormwater conveyance capacities and culvert dimensions to prevent urban flash flooding.

- Reservoir Rule Curves: Major storage dams in the Upper Chao Phraya Basin, notably Sirikit Dam (fed directly by the Nan River), will require dynamic flood-control buffer zones and adaptive rule curves to manage larger projected wet-season inflow pulses without increasing downstream flood risk in Central Thailand [24].

4.6 Study Limitations

Several methodological limitations should be recognized:

- Station Density: Although 14 stations provide comprehensive coverage of settled valleys and foothills, gauge density in inaccessible high-altitude ridges (>1,200 m) remains sparse, meaning high-elevation orographic intensification may be slightly underestimated.

- Spatial Interpolation: IDW interpolation represents mathematical distance-weighted interpolation across stations; it does not explicitly solve terrain-flow hydrodynamics like high-resolution regional climate models (RCMs) or numerical weather prediction tools.

- Ensemble Size: Seven GCMs under two scenarios capture a wide envelope of CMIP6 projections, but do not encompass the full 30+ model ensemble, though the selected models span low, medium, and high climate sensitivities.

- Sub-daily Extremes: The analysis is based on daily records. Sub-daily convective extremes (e.g., hourly intensities), which are critical for urban drainage and flash flood warnings, were not directly captured due to the lack of long-term continuous sub-daily historical gauge records.

5. Conclusions

This investigation delivered an empirical, multi-model assessment of historical and future extreme precipitation across Nan Province, Thailand, addressing the two core research questions:

- Historical Characteristics (RQ1): The observed 1981–2014 climate across the 14 gauges in Nan is characterized by substantial spatial gradients and intense interannual variability. Historical baseline network medians average 88.4 mm for Rx1day, 165.2 mm for Rx5day, 324.5 mm for R95p, and 96.3 mm for R99p. Flood-rich years (notably 2011, 2006, and 1994) exhibited extreme multi-day accumulations exceeding historical medians by 60–100%.

- Projected Future Changes (RQ2): Under CMIP6 multi-model projections for 2021–2050, extreme precipitation is projected to intensify province-wide under both SSP2-4.5 and SSP5-8.5. Multi-model ensemble (MME) medians project increases of +6.2% (Rx1day), +5.8% (Rx5day), +14.2% (R95p), and +18.5% (R99p) under SSP2-4.5, accelerating to +11.4%, +9.6%, +21.8%, and +29.4%, respectively, under SSP5-8.5. Threshold-based extreme totals (R95p and R99p) amplify at more than double the rate of maximum daily events. Geographically, projected intensifications are strongest across the eastern and northern mountainous districts (Bo Kluea, Pua, and Thung Chang), with inter-model agreement exceeding 71% to 100% across gauges.

These findings provide compelling, evidence-based guidance for regional water management authorities, infrastructure designers, and disaster mitigation agencies in Northern Thailand, demonstrating that engineering design standards and flood risk frameworks must be updated to accommodate intensifying rainfall extremes.

Acknowledgements

The authors gratefully acknowledge the Thai Meteorological Department (TMD) and the Royal Irrigation Department (RID) for providing observational rainfall and meteorological data for Nan Province. We also acknowledge the World Climate Research Programme's Working Group on Coupled Modelling for coordinating and producing the CMIP6 multi-model dataset.

References

- Intergovernmental Panel on Climate Change (IPCC). Climate Change 2021: The Physical Science Basis. Contribution of Working Group I to the Sixth Assessment Report of the Intergovernmental Panel on Climate Change. Cambridge: Cambridge University Press; 2021. Available from: https://doi.org/10.1017/9781009157896

- Dottori F, Szewczyk W, Ciscar JC, Zhao F, Alfieri L, Hirabayashi Y, et al. Increased human serious injury and flood risks in Europe under climate change. Nat Clim Chang. 2018;8(9):781-6. Available from: https://doi.org/10.1038/s41558-018-0257-2

- Loo YL, Billa L, Singh A. Effect of climate change on seasonal monsoon in Asia and its impact on the variability of monsoon rainfall in Southeast Asia. Geosci Front. 2015;6(6):817-23. Available from: https://doi.org/10.1016/j.gsf.2014.02.009

- Singhrattna N, Babel MS, Weesakul S. Spatial and temporal variation of extreme precipitation in Thailand. Int J Climatol. 2020;40(6):3142-60. Available from: https://doi.org/10.1002/joc.6388

- Komori D, Nakamura S, Kiguchi M, Nishijima A, Yamazaki D, Suzuki S, et al. Characteristics of the 2011 Chao Phraya River flood in Central Thailand. Hydrol Res Lett. 2012;6:41-6. Available from: https://doi.org/10.3178/hrl.6.41

- Eyring V, Bony S, Meehl GA, Senior CA, Stevens B, Stouffer RJ, et al. Overview of the Coupled Model Intercomparison Project Phase 6 (CMIP6) experimental design and organization. Geosci Model Dev. 2016;9(5):1937-58. Available from: https://doi.org/10.5194/gmd-9-1937-2016

- O’Neill BC, Tebaldi C, van Vuuren DP, Eyring V, Friedlingstein P, Hurtt G, et al. The Scenario Model Intercomparison Project (ScenarioMIP) for CMIP6. Geosci Model Dev. 2016;9(9):3461-82. Available from: https://doi.org/10.5194/gmd-9-3461-2016

- Zelinka MD, Myers TA, McCoy DT, Po-Chedley S, Caldwell PM, Ceppi P, et al. Causes of higher climate sensitivity in CMIP6 models. Geophys Res Lett. 2020;47(1):e2019GL085782. Available from: https://doi.org/10.1029/2019GL085782

- Tebaldi C, Debeire K, Eyring V, Fischer E, Fyfe J, Friedlingstein P, et al. Climate model projections from the Scenario Model Intercomparison Project (ScenarioMIP) of CMIP6. Earth Syst Dyn. 2021;12(1):253-93. Available from: https://doi.org/10.5194/esd-12-253-2021

- Tangang F, Chung JX, Juneng L, Supari, Salimun E, Ngai ST, et al. Projected future changes in mean precipitation and extreme events over Southeast Asia based on CORDEX-Southeast Asia ensemble simulations. Int J Climatol. 2020;40(6):3063-84. Available from: https://doi.org/10.1002/joc.6384

- Maraun D, Widmann M. Statistical Downscaling and Bias Correction for Climate Research. Cambridge: Cambridge University Press; 2018. Available from: https://doi.org/10.1017/9781107588783

- Cannon AJ, Sobie SR, Murdock TQ. Bias correction of GCM precipitation by quantile mapping: How well do methods preserve changes in quantiles and extremes? J Clim. 2015;28(17):6938-59. Available from: https://doi.org/10.1175/JCLI-D-14-00754.1

- Cannon AJ. Multivariate quantile mapping bias correction: an N-dimensional probability density function transform for climate simulations. J Clim. 2018;31(18):7461-81. Available from: https://doi.org/10.1175/JCLI-D-17-0610.1

- Piani C, Weedon GP, Best M, Gomes SM, Viterbo P, Hagemann S, et al. Statistical bias correction of global simulated daily precipitation and temperature for the calculation of hydrological impacts. J Hydrol. 2010;395(3-4):199-215. Available from: https://doi.org/10.1016/j.jhydrol.2010.10.024

- Zhang X, Alexander L, Hegerl GC, Jones P, Tank AK, Peterson TC, et al. Indices for monitoring changes in extremes based on daily temperature and precipitation data. Wiley Interdiscip Rev Clim Chang. 2011;2(6):851-70. Available from: https://doi.org/10.1002/wcc.147

- Ge F, Zhu S, Sielmann F, Fraedrich K, Zhi X, Zhu X, et al. Responses of extreme precipitation to global warming over Southeast Asia in CMIP5 models. Int J Climatol. 2019;39(6):2887-903. Available from: https://doi.org/10.1002/joc.6000

- Chen H, Sun J, Lin W, Xu H. Comparison of extreme precipitation indices between CMIP6 and CMIP5 models in China. Int J Climatol. 2021;41(1):501-18. Available from: https://doi.org/10.1002/joc.6644

- Champathong A, Komori D, Kiguchi M, Sukhapunnaphan T, Oki T. Future projection of flood inundation in the Chao Phraya River Basin using high-resolution regional climate models. J Disaster Res. 2013;8(3):414-23. Available from: https://doi.org/10.20965/jdr.2013.p0414

- Sharma D, Babel MS. Trends in extreme rainfall and temperature indices for the Upper Chao Phraya River Basin, Thailand. Int J Climatol. 2014;34(7):2393-407. Available from: https://doi.org/10.1002/joc.3846

- Trenberth KE, Dai A, Rasmussen RM, Parsons DB. The changing character of precipitation. Bull Am Meteorol Soc. 2003;84(9):1205-18. Available from: https://doi.org/10.1175/BAMS-84-9-1205

- Lenderink G, Van Meijgaard E. Increase in hourly precipitation extremes beyond expectations from temperature changes. Nat Geosci. 2008;1(8):511-4. Available from: https://doi.org/10.1038/ngeo262

- Roe GH. Orographic precipitation. Annu Rev Earth Planet Sci. 2005;33:645-71. Available from: https://doi.org/10.1146/annurev.earth.33.092203.122441

- Soralump S, Mairaing W, Tansathit K. Rainfall threshold for landslide early warning system in Thailand. Geotech Eng. 2010;41(3):1-9.

- Tebakari T, Yoshitani J, Suvanpimol C. Impact of climate change on hydrological regime in the Chao Phraya River Basin, Thailand. Hydrol Process. 2012;26(16):2452-60. Available from: https://doi.org/10.1002/hyp.8346