Supplementary Material

Climate-Signal Preservation of Bias-Corrected Daily Precipitation from Global Climate Models: A Comparison of Quantile Mapping Methods over Uttaradit, Thailand

Surasit Punyawansiri

Office of Water Management and Hydrology, Royal Irrigation Department, Dusit, Bangkok 10300, Thailand

Supplementary tables are numbered as they are cited in the manuscript. Where a listing runs to thousands of rows it is supplied as a machine-readable file with this submission rather than as a printed table; those files are listed at the end.

Supplementary Table S1. Design of the three bias-correction methods and the settings they share.

| Item | Description |
| --- | --- |
| Method: QM/EQM | core purpose: Historical bias correction; distribution treatment: Empirical marginal quantile mapping onto the observed calibration distribution; climate-signal property: Not explicitly preserved; temporal reconstruction: No |
| Method: DetQM | core purpose: Bias correction with mean-signal preservation; distribution treatment: Target-period wet-day mean removed multiplicatively, quantile-mapped, then restored; climate-signal property: Mean-signal oriented; temporal reconstruction: No |
| Method: QDM | core purpose: Bias correction with quantile-signal preservation; distribution treatment: Observed calibration quantile multiplied by the model's own quantile-level change ratio; climate-signal property: Quantile-signal oriented; temporal reconstruction: No |
| Wet-day threshold | 1.0 mm d⁻¹, identical for all methods |
| Zero-rain treatment | days at or below the frequency-adapted model threshold are set to zero; observed zeros are never imputed |
| Frequency adaptation | model wet-day threshold raised until the model wet-day frequency over the calibration period matches the observed frequency, fitted separately for each gauge-model pair |
| Empirical CDF | non-parametric, sorted wet-day sample |
| Plotting positions | hazen, (i − 0.5)/n |
| Interpolation | linear within the calibration quantile range |
| Extrapolation | QM and DetQM hold the multiplicative correction ratio of the nearest calibration quantile; QDM applies the model's own change ratio, which extends naturally beyond the range. No value is clipped. |
| Calibration period | 1981–2002, parameters frozen thereafter |
| Independent validation | 2003–2014, no refitting |
| Future baseline | 1995–2014, same model and same method as the future series |
| Future period | 2021–2050, recomputed from the daily corrected series |
| Minimum wet days to fit | 30 |
| Mean wet days extrapolated | 0.41% (maximum 10.6% for one gauge-model pair) |

Supplementary Table S2. Calibration-period mean absolute bias by index and method, averaged over 13 gauges and 7 GCMs. Station- and model-level values for all 16 indices are in Supplementary Data S2.

| Index | Raw CMIP6 | QM/EQM | DetQM | QDM |
| --- | --- | --- | --- | --- |
| Wet days | 71.14 | 0.03 | 0.46 | 0.03 |
| PRCPTOT | 40.82 | 1.19 | 1.2 | 1.19 |
| SDII | 45.25 | 0.01 | 0.42 | 0 |
| q50 | 46.92 | 0.02 | 0.63 | 0.02 |
| q90 | 49.93 | 0.01 | 0.23 | 0.01 |
| q95 | 48.17 | 0.02 | 0.19 | 0.02 |
| q99 | 51.57 | 0.08 | 0.15 | 0.08 |
| Rx1day | 49.11 | 7.27 | 7.27 | 7.27 |
| Rx5day | 34.52 | 25.52 | 25.52 | 25.52 |
| R10mm | 58.26 | 1.19 | 1.24 | 1.19 |
| R20mm | 74.42 | 1.38 | 1.46 | 1.36 |
| R50mm | 194.93 | 1.04 | 1.05 | 1.04 |
| R95p | 74.96 | 1.19 | 1.33 | 1.19 |
| R99p | 95.37 | 1.35 | 1.35 | 1.35 |
| CDD | 47.9 | 42.52 | 42.21 | 42.52 |
| CWD | 89.99 | 35.05 | 35.06 | 35.05 |

Supplementary Table S3. Independent-validation mean absolute bias by index and method, using parameters fitted on the calibration period and frozen. Station- and model-level values are in Supplementary Data S3.

| Index | Raw CMIP6 | QM/EQM | DetQM | QDM |
| --- | --- | --- | --- | --- |
| Wet days | 67.22 | 28.65 | 28.76 | 28.65 |
| PRCPTOT | 38.45 | 15.9 | 15.1 | 15.82 |
| SDII | 44.08 | 26.91 | 26.97 | 27.37 |
| q50 | 43.2 | 24.17 | 26.31 | 26.52 |
| q90 | 47.24 | 32.31 | 32.86 | 33.36 |
| q95 | 47.95 | 31.17 | 31.93 | 33.69 |
| q99 | 54.81 | 28.8 | 27.39 | 27.61 |
| Rx1day | 47.38 | 30.96 | 29.98 | 26.85 |
| Rx5day | 35.63 | 33.59 | 33.08 | 31.52 |
| R10mm | 53.39 | 17.79 | 17.47 | 18.75 |
| R20mm | 70.25 | 35.95 | 36.12 | 37.27 |
| R50mm | 124.84 | 60.71 | 61.48 | 61.46 |
| R95p | 74.16 | 41.81 | 43.51 | 45.48 |
| R99p | 88.64 | 58.74 | 60.59 | 60.76 |
| CDD | 48.42 | 51.78 | 51.98 | 51.47 |
| CWD | 83.19 | 38.4 | 38.48 | 38.73 |

Supplementary Table S4. Climate-signal preservation summary for every scenario, index and method.

| Scenario | Index | Method | n | Gauges | Median PE (%) | IQR (%) | Mean (%) | SD (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSP2-4.5 | Wet days | QM/EQM | 7 | 13 | 0.29 | -2.02 to 2.22 | 0.06 | 3.27 |
| SSP2-4.5 | Wet days | DetQM | 7 | 13 | 0.84 | -1.77 to 2.35 | 0.41 | 3.59 |
| SSP2-4.5 | Wet days | QDM | 7 | 13 | -0.09 | -2.18 to 2.50 | -0.04 | 3.35 |
| SSP2-4.5 | PRCPTOT | QM/EQM | 7 | 13 | 1.26 | 0.30 to 3.78 | 1.5 | 2.62 |
| SSP2-4.5 | PRCPTOT | DetQM | 7 | 13 | 0.81 | -0.23 to 2.25 | 0.47 | 2.41 |
| SSP2-4.5 | PRCPTOT | QDM | 7 | 13 | 0.73 | -0.18 to 2.12 | 0.97 | 1.74 |
| SSP2-4.5 | SDII | QM/EQM | 7 | 13 | 0.94 | 0.32 to 2.77 | 2 | 4.94 |
| SSP2-4.5 | SDII | DetQM | 7 | 13 | -0.09 | -1.34 to 1.64 | 0.61 | 4.94 |
| SSP2-4.5 | SDII | QDM | 7 | 13 | 0.69 | -1.04 to 2.13 | 1.57 | 4.63 |
| SSP2-4.5 | q50 | QM/EQM | 7 | 13 | 4.18 | 0.72 to 7.36 | 3.96 | 5.04 |
| SSP2-4.5 | q50 | DetQM | 7 | 13 | 0.88 | -1.36 to 5.78 | 1.32 | 6.4 |
| SSP2-4.5 | q50 | QDM | 7 | 13 | 1.22 | 0.89 to 6.34 | 3.25 | 4.58 |
| SSP2-4.5 | q90 | QM/EQM | 7 | 13 | 0.16 | -0.12 to 3.48 | 3.12 | 6.47 |
| SSP2-4.5 | q90 | DetQM | 7 | 13 | 1.03 | -0.81 to 3.34 | 2.53 | 5.64 |
| SSP2-4.5 | q90 | QDM | 7 | 13 | 0.43 | -0.78 to 3.63 | 2.84 | 6.26 |
| SSP2-4.5 | q95 | QM/EQM | 7 | 13 | 0.09 | -1.80 to 2.03 | 1.17 | 4.16 |
| SSP2-4.5 | q95 | DetQM | 7 | 13 | -0.02 | -1.43 to 1.63 | 0.74 | 2.98 |
| SSP2-4.5 | q95 | QDM | 7 | 13 | -0.05 | -1.03 to 1.29 | 0.78 | 2.92 |
| SSP2-4.5 | q99 | QM/EQM | 7 | 13 | 1.96 | -0.83 to 2.93 | 0.73 | 11.27 |
| SSP2-4.5 | q99 | DetQM | 7 | 13 | 1.46 | -1.29 to 2.11 | 0.17 | 9.73 |
| SSP2-4.5 | q99 | QDM | 7 | 13 | 0.26 | -2.21 to 2.08 | 0.26 | 8.32 |
| SSP2-4.5 | Rx1day | QM/EQM | 7 | 13 | 1.63 | 0.34 to 1.97 | -0.86 | 6.29 |
| SSP2-4.5 | Rx1day | DetQM | 7 | 13 | 0.81 | -0.57 to 1.21 | -1.24 | 5.09 |
| SSP2-4.5 | Rx1day | QDM | 7 | 13 | 0.59 | -0.47 to 1.13 | -0.06 | 2.69 |
| SSP2-4.5 | R10mm | QM/EQM | 7 | 13 | -0.8 | -2.34 to 4.77 | 1.22 | 5.54 |
| SSP2-4.5 | R10mm | DetQM | 7 | 13 | -2.44 | -3.05 to 1.56 | -0.56 | 5 |
| SSP2-4.5 | R10mm | QDM | 7 | 13 | -2.03 | -2.42 to 1.62 | -0.05 | 4.34 |
| SSP2-4.5 | R20mm | QM/EQM | 7 | 13 | 1 | -1.72 to 4.87 | 1.17 | 14.28 |
| SSP2-4.5 | R20mm | DetQM | 7 | 13 | 0.52 | 0.14 to 3.12 | 1.49 | 13.58 |
| SSP2-4.5 | R20mm | QDM | 7 | 13 | 2.11 | -0.77 to 6.09 | 3.75 | 9.26 |
| SSP2-4.5 | R50mm | QM/EQM | 6 | 9–12 | 15.54 | -4.83 to 25.34 | 16.18 | 57.65 |
| SSP2-4.5 | R50mm | DetQM | 6 | 9–12 | 6.6 | 1.90 to 23.87 | 12.32 | 50.94 |
| SSP2-4.5 | R50mm | QDM | 6 | 9–12 | 14.76 | 10.55 to 18.52 | 14.98 | 48.03 |
| SSP2-4.5 | R95p | QM/EQM | 7 | 6–13 | -1.49 | -3.43 to 2.32 | 1.55 | 29.97 |
| SSP2-4.5 | R95p | DetQM | 7 | 6–13 | -2.59 | -4.10 to 0.81 | -0.56 | 24.85 |
| SSP2-4.5 | R95p | QDM | 7 | 6–13 | -1.45 | -3.95 to 0.08 | 1.07 | 27.97 |
| SSP2-4.5 | R99p | QM/EQM | 7 | 5–13 | -9.21 | -19.36 to 3.38 | -6.33 | 18.99 |
| SSP2-4.5 | R99p | DetQM | 7 | 5–13 | -5.76 | -9.09 to -0.23 | -3.35 | 20.5 |
| SSP2-4.5 | R99p | QDM | 7 | 5–13 | -5.95 | -14.19 to 18.91 | 5.45 | 34.4 |
| SSP2-4.5 | Rx5day | QM/EQM | 7 | 13 | 0.83 | 0.31 to 1.71 | -0.12 | 6.71 |
| SSP2-4.5 | Rx5day | DetQM | 7 | 13 | 1.09 | -1.42 to 1.38 | -0.71 | 5.56 |
| SSP2-4.5 | Rx5day | QDM | 7 | 13 | -0.15 | -1.51 to 0.66 | -0.18 | 4.47 |
| SSP2-4.5 | CDD | QM/EQM | 7 | 13 | -2.41 | -5.74 to 0.30 | -1.38 | 5.78 |
| SSP2-4.5 | CDD | DetQM | 7 | 13 | -3.1 | -6.24 to 0.69 | -1.68 | 6.18 |
| SSP2-4.5 | CDD | QDM | 7 | 13 | -3.32 | -5.44 to 1.26 | -1.07 | 6.22 |
| SSP2-4.5 | CWD | QM/EQM | 7 | 13 | 2.8 | -1.97 to 6.70 | 2.6 | 6.25 |
| SSP2-4.5 | CWD | DetQM | 7 | 13 | 2.79 | -2.11 to 6.71 | 2.9 | 6.53 |
| SSP2-4.5 | CWD | QDM | 7 | 13 | 1.73 | -1.89 to 7.08 | 2.79 | 6.62 |
| SSP5-8.5 | Wet days | QM/EQM | 7 | 13 | -0.97 | -1.75 to -0.11 | -0.79 | 1.43 |
| SSP5-8.5 | Wet days | DetQM | 7 | 13 | -1.25 | -1.62 to -0.07 | -0.73 | 1.49 |
| SSP5-8.5 | Wet days | QDM | 7 | 13 | -1.48 | -1.74 to -0.11 | -0.74 | 1.59 |
| SSP5-8.5 | PRCPTOT | QM/EQM | 7 | 13 | 1.01 | -0.23 to 2.05 | 2.17 | 4.14 |
| SSP5-8.5 | PRCPTOT | DetQM | 7 | 13 | 1.19 | -0.04 to 1.72 | 1.73 | 2.85 |
| SSP5-8.5 | PRCPTOT | QDM | 7 | 13 | -0.07 | -0.36 to 0.64 | 1.2 | 3.27 |
| SSP5-8.5 | SDII | QM/EQM | 7 | 13 | 1.46 | 0.08 to 3.06 | 3.07 | 5.93 |
| SSP5-8.5 | SDII | DetQM | 7 | 13 | 1.17 | 0.22 to 2.17 | 2.52 | 4.32 |
| SSP5-8.5 | SDII | QDM | 7 | 13 | 1.12 | 0.09 to 1.86 | 2.08 | 4.78 |
| SSP5-8.5 | q50 | QM/EQM | 7 | 13 | 2.33 | 2.16 to 3.79 | 3.38 | 3.13 |
| SSP5-8.5 | q50 | DetQM | 7 | 13 | 1.88 | 0.81 to 4.05 | 2.62 | 3.17 |
| SSP5-8.5 | q50 | QDM | 7 | 13 | 1.84 | 0.43 to 3.63 | 2.76 | 3.25 |
| SSP5-8.5 | q90 | QM/EQM | 7 | 13 | 1.21 | -2.05 to 4.03 | 4.37 | 11.66 |
| SSP5-8.5 | q90 | DetQM | 7 | 13 | 1.46 | -0.80 to 4.21 | 4.01 | 8.76 |
| SSP5-8.5 | q90 | QDM | 7 | 13 | 1.7 | -0.72 to 3.97 | 4.15 | 9.42 |
| SSP5-8.5 | q95 | QM/EQM | 7 | 13 | 2.72 | 1.79 to 5.13 | 3.32 | 4.28 |
| SSP5-8.5 | q95 | DetQM | 7 | 13 | 2.78 | 1.72 to 3.17 | 2.2 | 2.46 |
| SSP5-8.5 | q95 | QDM | 7 | 13 | 0.76 | 0.51 to 2.69 | 1.26 | 2.39 |
| SSP5-8.5 | q99 | QM/EQM | 7 | 13 | 3.6 | -1.32 to 7.53 | 4.42 | 7.41 |
| SSP5-8.5 | q99 | DetQM | 7 | 13 | 3.71 | -1.19 to 6.64 | 3.59 | 5.75 |
| SSP5-8.5 | q99 | QDM | 7 | 13 | 0.04 | -1.72 to 2.99 | 0.83 | 3.61 |
| SSP5-8.5 | Rx1day | QM/EQM | 7 | 13 | 0.82 | 0.64 to 3.13 | 1.98 | 2.08 |
| SSP5-8.5 | Rx1day | DetQM | 7 | 13 | 1.34 | 0.58 to 3.15 | 2.15 | 2.17 |
| SSP5-8.5 | Rx1day | QDM | 7 | 13 | 0.03 | -0.71 to 1.10 | 0.16 | 1.78 |
| SSP5-8.5 | R10mm | QM/EQM | 7 | 13 | 3.71 | -0.56 to 6.88 | 5.17 | 10.74 |
| SSP5-8.5 | R10mm | DetQM | 7 | 13 | 4.26 | -1.42 to 8.18 | 4.62 | 7.57 |
| SSP5-8.5 | R10mm | QDM | 7 | 13 | 0.48 | -0.04 to 6.51 | 4.56 | 8.02 |
| SSP5-8.5 | R20mm | QM/EQM | 7 | 13 | 6.53 | 4.40 to 10.37 | 6.72 | 5.86 |
| SSP5-8.5 | R20mm | DetQM | 7 | 13 | 5.24 | 2.22 to 7.93 | 5.46 | 5.88 |
| SSP5-8.5 | R20mm | QDM | 7 | 13 | 5.73 | -1.03 to 9.46 | 4.28 | 7.25 |
| SSP5-8.5 | R50mm | QM/EQM | 6 | 9–12 | 5.29 | 4.03 to 13.12 | 14.59 | 31.34 |
| SSP5-8.5 | R50mm | DetQM | 6 | 9–12 | 9.67 | -0.79 to 20.26 | 11.69 | 22.67 |
| SSP5-8.5 | R50mm | QDM | 6 | 9–12 | 8.38 | -10.86 to 18.38 | 5.19 | 19.62 |
| SSP5-8.5 | R95p | QM/EQM | 7 | 6–13 | -2.85 | -6.95 to 6.04 | 2.9 | 17.08 |
| SSP5-8.5 | R95p | DetQM | 7 | 6–13 | -1.74 | -4.06 to 6.80 | 1.54 | 9.31 |
| SSP5-8.5 | R95p | QDM | 7 | 6–13 | -6.21 | -8.23 to -3.05 | -6.49 | 5.47 |
| SSP5-8.5 | R99p | QM/EQM | 7 | 5–13 | 13.86 | -18.45 to 17.46 | 4.84 | 28.25 |
| SSP5-8.5 | R99p | DetQM | 7 | 5–13 | 4.75 | -14.57 to 24.42 | 5.14 | 29.75 |
| SSP5-8.5 | R99p | QDM | 7 | 5–13 | -5.98 | -15.71 to 21.18 | 5.18 | 34.97 |
| SSP5-8.5 | Rx5day | QM/EQM | 7 | 13 | 1.02 | -0.19 to 4.74 | 2.22 | 3.14 |
| SSP5-8.5 | Rx5day | DetQM | 7 | 13 | 1.36 | -0.42 to 3.17 | 1.8 | 2.66 |
| SSP5-8.5 | Rx5day | QDM | 7 | 13 | -0.3 | -0.65 to 1.35 | 0.22 | 1.42 |
| SSP5-8.5 | CDD | QM/EQM | 7 | 13 | 2.58 | -3.68 to 7.09 | 1.05 | 9.21 |
| SSP5-8.5 | CDD | DetQM | 7 | 13 | 2.61 | -3.29 to 6.82 | 1.04 | 9.34 |
| SSP5-8.5 | CDD | QDM | 7 | 13 | 2.25 | -3.03 to 6.77 | 0.92 | 9.2 |
| SSP5-8.5 | CWD | QM/EQM | 7 | 13 | -1.97 | -4.50 to 3.64 | 0.25 | 9.08 |
| SSP5-8.5 | CWD | DetQM | 7 | 13 | -1.61 | -4.60 to 3.79 | 0.36 | 9.38 |
| SSP5-8.5 | CWD | QDM | 7 | 13 | -1.56 | -4.17 to 3.52 | 0.58 | 9.47 |

Supplementary Data S5. Climate-signal preservation for every model, gauge, scenario and index. This listing has 9,804 rows and is supplied as a machine-readable file with this submission rather than as a printed table.

| Contents | Rows | Columns | File |
| --- | --- | --- | --- |
| signal-preservation error per model, gauge, scenario and index | 9804 | 11 | Supplementary_Data_S5.csv |

Supplementary Table S6. Descriptive two-factor sum-of-squares partition for all indices and both scenarios.

| Scenario | Index | N cells | GCMs | GCM main effect (%) | BC-method main effect (%) | Residual (%) |
| --- | --- | --- | --- | --- | --- | --- |
| SSP2-4.5 | CDD | 273 | 7 | 49.9 | 0.1 | 50 |
| SSP2-4.5 | CWD | 273 | 7 | 52.9 | 0 | 47 |
| SSP2-4.5 | PRCPTOT | 273 | 7 | 78.3 | 0.6 | 21.1 |
| SSP2-4.5 | PRCPTOT_dry | 273 | 7 | 77.4 | 0.1 | 22.6 |
| SSP2-4.5 | PRCPTOT_wet | 273 | 7 | 67.8 | 0.6 | 31.6 |
| SSP2-4.5 | R10mm | 273 | 7 | 73.5 | 0.9 | 25.6 |
| SSP2-4.5 | R20mm | 273 | 7 | 9.5 | 0.3 | 90.2 |
| SSP2-4.5 | R50mm | 214 | 7 | 18.5 | 0.2 | 81.3 |
| SSP2-4.5 | R95p | 273 | 7 | 66.9 | 0.1 | 33 |
| SSP2-4.5 | R99p | 272 | 7 | 30.5 | 1.1 | 68.4 |
| SSP2-4.5 | Rx1day | 273 | 7 | 82.3 | 0.1 | 17.6 |
| SSP2-4.5 | Rx5day | 273 | 7 | 75.6 | 0.1 | 24.4 |
| SSP2-4.5 | SDII | 273 | 7 | 63.6 | 1.6 | 34.9 |
| SSP2-4.5 | q50 | 273 | 7 | 25.4 | 5.4 | 69.2 |
| SSP2-4.5 | q90 | 273 | 7 | 45 | 0.2 | 54.8 |
| SSP2-4.5 | q95 | 273 | 7 | 67.4 | 0 | 32.5 |
| SSP2-4.5 | q99 | 273 | 7 | 70.6 | 0 | 29.4 |
| SSP2-4.5 | wet_day_pct | 273 | 7 | 64 | 0.2 | 35.8 |
| SSP5-8.5 | CDD | 273 | 7 | 58.8 | 0 | 41.2 |
| SSP5-8.5 | CWD | 273 | 7 | 73.8 | 0 | 26.1 |
| SSP5-8.5 | PRCPTOT | 273 | 7 | 83.5 | 0.2 | 16.3 |
| SSP5-8.5 | PRCPTOT_dry | 273 | 7 | 77.9 | 0 | 22 |
| SSP5-8.5 | PRCPTOT_wet | 273 | 7 | 81.8 | 0.2 | 18.1 |
| SSP5-8.5 | R10mm | 273 | 7 | 76.2 | 0 | 23.8 |
| SSP5-8.5 | R20mm | 273 | 7 | 42.2 | 0 | 57.8 |
| SSP5-8.5 | R50mm | 214 | 7 | 25.4 | 0 | 74.5 |
| SSP5-8.5 | R95p | 273 | 7 | 80.2 | 1 | 18.9 |
| SSP5-8.5 | R99p | 272 | 7 | 74.5 | 0.1 | 25.4 |
| SSP5-8.5 | Rx1day | 273 | 7 | 74.9 | 0.2 | 24.8 |
| SSP5-8.5 | Rx5day | 273 | 7 | 74.1 | 0.2 | 25.7 |
| SSP5-8.5 | SDII | 273 | 7 | 76.8 | 0.3 | 23 |
| SSP5-8.5 | q50 | 273 | 7 | 52.5 | 0.4 | 47 |
| SSP5-8.5 | q90 | 273 | 7 | 72.6 | 0 | 27.4 |
| SSP5-8.5 | q95 | 273 | 7 | 70.6 | 0.5 | 29 |
| SSP5-8.5 | q99 | 273 | 7 | 68.6 | 0.7 | 30.7 |
| SSP5-8.5 | wet_day_pct | 273 | 7 | 78.2 | 0 | 21.8 |

Supplementary Table S7. Temporal-dependence statistics: mean absolute bias against observations, averaged over gauges and GCMs. Gauge- and model-level values are in Supplementary Data S7.

| Period | Method | ACF(1) daily | ACF(1) occurrence | P(wet ¦ wet) | P(wet ¦ dry) | Mean wet spell | Mean dry spell | p90 dry spell |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Calibration | Raw CMIP6 | 118.14 | 29.78 | 30.44 | 68.48 | 64.98 | 46.55 | 48.24 |
| Calibration | QM/EQM | 110.15 | 32.42 | 13.98 | 56.37 | 30.44 | 31.11 | 44.7 |
| Calibration | DetQM | 110.15 | 32.41 | 13.99 | 56.18 | 30.46 | 31.33 | 44.97 |
| Calibration | QDM | 110.15 | 32.42 | 13.98 | 56.37 | 30.44 | 31.11 | 44.7 |
| Validation | Raw CMIP6 | 82.77 | 28.43 | 26.99 | 74.76 | 53.35 | 54.26 | 56.28 |
| Validation | QM/EQM | 74.73 | 29.79 | 14.25 | 62.55 | 28.11 | 35.66 | 37.37 |
| Validation | DetQM | 75.15 | 29.81 | 14.32 | 62.6 | 28.19 | 35.97 | 37.72 |
| Validation | QDM | 74.43 | 29.79 | 14.4 | 62.16 | 28.23 | 36.43 | 38.35 |

Supplementary Table S8. Paired method differences against QM/EQM, model-level and descriptive.

| Scenario | Index | DetQM − QM/EQM median (%) | n paired GCMs | QDM − QM/EQM median (%) |
| --- | --- | --- | --- | --- |
| SSP2-4.5 | Wet days | 0.01 | 7 | -0.28 |
| SSP2-4.5 | PRCPTOT | -1.11 | 7 | -0.65 |
| SSP2-4.5 | SDII | -1.54 | 7 | -0.66 |
| SSP2-4.5 | q50 | -2.69 | 7 | -0.53 |
| SSP2-4.5 | q90 | -0.29 | 7 | 0.08 |
| SSP2-4.5 | q95 | -0.2 | 7 | -0.45 |
| SSP2-4.5 | q99 | -0.25 | 7 | -0.95 |
| SSP2-4.5 | Rx1day | -0.41 | 7 | -0.21 |
| SSP2-4.5 | R10mm | -1.63 | 7 | -0.99 |
| SSP2-4.5 | R20mm | 0.49 | 7 | 0.08 |
| SSP2-4.5 | R50mm | -4.24 | 6 | -2.11 |
| SSP2-4.5 | R95p | -1.44 | 7 | -1.04 |
| SSP2-4.5 | R99p | 0.35 | 7 | -1.22 |
| SSP2-4.5 | Rx5day | -0.79 | 7 | -0.98 |
| SSP2-4.5 | CDD | -0.06 | 7 | 0.75 |
| SSP2-4.5 | CWD | 0 | 7 | 0.21 |
| SSP5-8.5 | Wet days | 0.01 | 7 | 0.06 |
| SSP5-8.5 | PRCPTOT | -0.21 | 7 | -0.71 |
| SSP5-8.5 | SDII | -0.29 | 7 | -0.34 |
| SSP5-8.5 | q50 | -1.28 | 7 | -0.48 |
| SSP5-8.5 | q90 | 0.21 | 7 | 0.37 |
| SSP5-8.5 | q95 | -0.36 | 7 | -0.27 |
| SSP5-8.5 | q99 | -0.12 | 7 | -1.28 |
| SSP5-8.5 | Rx1day | 0.39 | 7 | -1.39 |
| SSP5-8.5 | R10mm | 0.27 | 7 | -0.72 |
| SSP5-8.5 | R20mm | 1.39 | 7 | 0.12 |
| SSP5-8.5 | R50mm | -0.19 | 6 | -3.21 |
| SSP5-8.5 | R95p | 0.3 | 7 | -1.59 |
| SSP5-8.5 | R99p | -2.07 | 7 | 3.11 |
| SSP5-8.5 | Rx5day | -0.28 | 7 | -0.89 |
| SSP5-8.5 | CDD | 0.03 | 7 | 0.22 |
| SSP5-8.5 | CWD | 0 | 7 | 0.05 |

Supplementary Table S9. Acceptance-gate status.

| gate | status | criterion | evidence | limitation |
| --- | --- | --- | --- | --- |
| A: observed data | PASS WITH RESIDUAL BIAS | suspicious zeros classified by automated screening; no imputation | classes {'not_zero': 4866, 'confirmed_zero': 315, 'uncertain': 80, 'probable_missing': 43}; 0 values imputed | station metadata not yet verified; 4 gauge(s) inhomogeneous between calibration and validation (351003, 351005, 351006, 351007). QC is closed for automated screening but NOT for station homogenisation. |
| B: historical bias correction | PASS (MARGINAL ONLY) | calibration-period MARGINAL distribution reproduced (¦bias¦ < 5%); this criterion covers the marginal distribution only and makes no claim about temporal or multi-day structure (see Gate G) | marginal ¦bias¦ 0.19%; spell/multi-day ¦bias¦ 27.59% | quantile mapping is a marginal transformation and does not reorder days, so CDD, CWD and Rx5day retain the model's own temporal structure. Do not claim that extremes as a class were successfully reproduced. |
| C: independent validation | PASS WITH RESIDUAL BIAS | out-of-sample ¦bias¦ smaller than raw GCM on the primary metrics | improved on 4/4; mean residual ¦bias¦ 26.4%; excluding the gauges flagged as non-homogeneous, the network-mean residual is 13.4% | residual out-of-sample error remains substantial. 'Better than raw' does not mean 'reliable for all extremes'; station-level errors are larger than the network mean. The reduction obtained by excluding non-homogeneous gauges is an association between station heterogeneity and validation error, not a causal attribution of a share of that error. |
| D: future signal | PASS | change = BC_future / BC_historical, same model and method | enforced in code; observations never used as denominator |  |
| E: method comparison | PASS | raw, QM, DetQM and QDM evaluated on an identical split | methods present: ['detqm', 'qdm', 'qm', 'raw'] |  |
| F: maps and boundary | PASS | verified administrative boundary, CRS and coordinates | verified |  |
| G: temporal dependence | DIAGNOSTIC PASS - TEMPORAL STRUCTURE NOT CORRECTED (NOT A PREDICTION-PERFORMANCE PASS) | DIAGNOSTIC ONLY - this gate measures and reports temporal dependence; it is not an acceptance criterion for prediction performance. No hard threshold is imposed because QM/DetQM/QDM are marginal transformations and do not explicitly reconstruct temporal sequencing | improved vs raw on 3/4 sequencing statistics (P_wet_given_wet, mean_dry_spell, mean_wet_spell); worst residual ¦bias¦ 33.77%; not improved: acf1_occurrence | the bias correction improved selected occurrence-transition statistics relative to the raw GCM, but substantial residual biases remain in spell-duration and multi-day statistics. Do not interpret CDD, CWD or Rx5day projections as corrected. |
| H: raw-data anomaly review | PASS | raw model change reviewed against the regional literature range; flagged cases are quarantined from interpretation, not failed | no anomaly ¦ source trace: B3 NetCDF metadata=NOT ESTABLISHED; B4 unit conversion=ESTABLISHED (indirect); B5 calendar=ESTABLISHED; B6 concatenation=ESTABLISHED; B7 grid/station extraction=PARTIAL; B8 raw-only signal=ESTABLISHED |  |

Supplementary Table S10. CMIP6 model inventory.

| model | scenario | variant | grid | file | calendar | days_per_year | first | last | n_days | mean_daily_mm | implied_annual_mm | verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACCESS-ESM1-5 | historical | r1i1p1f1 | gn | pr_day_ACCESS-ESM1-5_historical_r1i1p1f1_gn_19810101-20141231_63sta.csv | standard (Gregorian) | [np.int64(365), np.int64(366)] | 1981-01-01 | 2014-12-31 | 12418 | 1.5009 | 548.2 | OK: consistent with mm/day |
| ACCESS-ESM1-5 | ssp245 | r1i1p1f1 | gn | pr_day_ACCESS-ESM1-5_ssp245_r1i1p1f1_gn_20150101-21001231_63sta.csv | standard (Gregorian) | [np.int64(365), np.int64(366)] | 2015-01-01 | 2100-12-31 | 31411 | 1.5228 | 556.2 | OK: consistent with mm/day |
| ACCESS-ESM1-5 | ssp585 | r1i1p1f1 | gn | pr_day_ACCESS-ESM1-5_ssp585_r1i1p1f1_gn_20150101-21001231_63sta.csv | standard (Gregorian) | [np.int64(365), np.int64(366)] | 2015-01-01 | 2100-12-31 | 31411 | 1.226 | 447.8 | OK: consistent with mm/day |
| CESM2 | historical | r11i1p1f1 | gn | pr_day_CESM2_historical_r11i1p1f1_gn_19810101-20141231_63sta.csv | 365_day (noleap) | [np.int64(365)] | 1981-01-01 | 2014-12-31 | 12410 | 2.8099 | 1026.3 | OK: consistent with mm/day |
| CESM2 | ssp245 | r11i1p1f1 | gn | pr_day_CESM2_ssp245_r11i1p1f1_gn_20150101-21001231_63sta.csv | 365_day (noleap) | [np.int64(365)] | 2015-01-01 | 2100-12-31 | 31390 | 2.9056 | 1061.3 | OK: consistent with mm/day |
| CESM2 | ssp585 | r11i1p1f1 | gn | pr_day_CESM2_ssp585_r11i1p1f1_gn_20150101-21001231_63sta.csv | 365_day (noleap) | [np.int64(365)] | 2015-01-01 | 2100-12-31 | 31390 | 2.5124 | 917.6 | OK: consistent with mm/day |
| CanESM5 | historical | r1i1p1f1 | gn | pr_day_CanESM5_historical_r1i1p1f1_gn_19810101-20141231_63sta.csv | 365_day (noleap) | [np.int64(365)] | 1981-01-01 | 2014-12-31 | 12410 | 4.2149 | 1539.5 | OK: consistent with mm/day |
| CanESM5 | ssp245 | r1i1p1f1 | gn | pr_day_CanESM5_ssp245_r1i1p1f1_gn_20150101-21001231_63sta.csv | 365_day (noleap) | [np.int64(365)] | 2015-01-01 | 2100-12-31 | 31390 | 4.0529 | 1480.3 | OK: consistent with mm/day |
| CanESM5 | ssp585 | r1i1p1f1 | gn | pr_day_CanESM5_ssp585_r1i1p1f1_gn_20150101-21001231_63sta.csv | 365_day (noleap) | [np.int64(365)] | 2015-01-01 | 2100-12-31 | 31390 | 3.3705 | 1231.1 | OK: consistent with mm/day |
| EC-Earth3 | historical | r1i1p1f1 | gr | pr_day_EC-Earth3_historical_r1i1p1f1_gr_19810101-20141231_63sta.csv | standard (Gregorian) | [np.int64(365), np.int64(366)] | 1981-01-01 | 2014-12-31 | 12418 | 1.3686 | 499.9 | OK: consistent with mm/day |
| EC-Earth3 | ssp245 | r1i1p1f1 | gr | pr_day_EC-Earth3_ssp245_r1i1p1f1_gr_20150101-21001231_63sta.csv | standard (Gregorian) | [np.int64(365), np.int64(366)] | 2015-01-01 | 2100-12-31 | 31411 | 1.5231 | 556.3 | OK: consistent with mm/day |
| EC-Earth3 | ssp585 | r1i1p1f1 | gr | pr_day_EC-Earth3_ssp585_r1i1p1f1_gr_20150101-21001231_63sta.csv | standard (Gregorian) | [np.int64(365), np.int64(366)] | 2015-01-01 | 2100-12-31 | 31411 | 1.4538 | 531 | OK: consistent with mm/day |
| FGOALS-g3 | historical | r1i1p1f1 | gn | pr_day_FGOALS-g3_historical_r1i1p1f1_gn_19810101-20141231_63sta.csv | 365_day (noleap) | [np.int64(365)] | 1981-01-01 | 2014-12-31 | 12410 | 0.7866 | 287.3 | OK: consistent with mm/day |
| FGOALS-g3 | ssp245 | r1i1p1f1 | gn | pr_day_FGOALS-g3_ssp245_r1i1p1f1_gn_20150101-21001231_63sta.csv | 365_day (noleap) | [np.int64(365)] | 2015-01-01 | 2100-12-31 | 31390 | 0.6907 | 252.3 | OK: consistent with mm/day |
| FGOALS-g3 | ssp585 | r1i1p1f1 | gn | pr_day_FGOALS-g3_ssp585_r1i1p1f1_gn_20150101-21001231_63sta.csv | 365_day (noleap) | [np.int64(365)] | 2015-01-01 | 2100-12-31 | 31390 | 0.6254 | 228.4 | OK: consistent with mm/day |
| MIROC6 | historical | r1i1p1f1 | gn | pr_day_MIROC6_historical_r1i1p1f1_gn_19810101-20141231_63sta.csv | standard (Gregorian) | [np.int64(365), np.int64(366)] | 1981-01-01 | 2014-12-31 | 12418 | 3.0912 | 1129.1 | OK: consistent with mm/day |
| MIROC6 | ssp245 | r1i1p1f1 | gn | pr_day_MIROC6_ssp245_r1i1p1f1_gn_20150101-21001231_63sta.csv | standard (Gregorian) | [np.int64(365), np.int64(366)] | 2015-01-01 | 2100-12-31 | 31411 | 3.1708 | 1158.1 | OK: consistent with mm/day |
| MIROC6 | ssp585 | r1i1p1f1 | gn | pr_day_MIROC6_ssp585_r1i1p1f1_gn_20150101-21001231_63sta.csv | standard (Gregorian) | [np.int64(365), np.int64(366)] | 2015-01-01 | 2100-12-31 | 31411 | 3.046 | 1112.6 | OK: consistent with mm/day |
| MRI-ESM2-0 | historical | r1i1p1f1 | gn | pr_day_MRI-ESM2-0_historical_r1i1p1f1_gn_19810101-20141231_63sta.csv | standard (Gregorian) | [np.int64(365), np.int64(366)] | 1981-01-01 | 2014-12-31 | 12418 | 2.4819 | 906.5 | OK: consistent with mm/day |
| MRI-ESM2-0 | ssp245 | r1i1p1f1 | gn | pr_day_MRI-ESM2-0_ssp245_r1i1p1f1_gn_20150101-21001231_63sta.csv | standard (Gregorian) | [np.int64(365), np.int64(366)] | 2015-01-01 | 2100-12-31 | 31411 | 2.2751 | 831 | OK: consistent with mm/day |
| MRI-ESM2-0 | ssp585 | r1i1p1f1 | gn | pr_day_MRI-ESM2-0_ssp585_r1i1p1f1_gn_20150101-21001231_63sta.csv | standard (Gregorian) | [np.int64(365), np.int64(366)] | 2015-01-01 | 2100-12-31 | 31411 | 1.8167 | 663.5 | OK: consistent with mm/day |

Supplementary Data files

The complete listings behind the aggregated tables above are provided as comma-separated files in this submission.

| File | Rows | Columns |
| --- | --- | --- |
| Supplementary_Data_S2.csv | 364 | 67 |
| Supplementary_Data_S3.csv | 364 | 67 |
| Supplementary_Data_S5.csv | 9804 | 11 |
| Supplementary_Data_S7.csv | 728 | 43 |
| Supplementary_Data_S10b_fit_diagnostics.csv | 91 | 9 |
| Supplementary_Data_S10c_apply_diagnostics.csv | 546 | 10 |