Observed Baseline Climatology, Extreme Precipitation Indices, and CMIP6 Projections over Uttaradit Province, Thailand

Target: Scopus Q1–Q2 Climate Journal | Study Area: Uttaradit Province | Baseline: 1995–2014 | Projections: 2021–2050

# Abstract

Accurate assessment of precipitation extremes and their future projections is critical for resilient water resources management in the Upper Nan River basin and the operational security of the Sirikit Dam in Uttaradit Province, Northern Thailand. This study establishes a rigorous baseline climatology of daily precipitation and 11 Expert Team on Climate Change Detection and Indices (ETCCDI) extreme metrics across 13 meteorological stations for the authoritative 1995–2014 period, and evaluates the performance and future projections of seven Coupled Model Intercomparison Project Phase 6 (CMIP6) General Circulation Models (GCMs). Baseline mean annual precipitation across Uttaradit averaged 1139.80 ± 213.19 mm (ranging from 947.71 mm at Station 351012 to 1425.06 mm at Station 351003), with 77.8% (886.85 mm) concentrated during the Southwest Monsoon wet season (May–October). Pre-computed Quantile Delta Mapping (QDM) bias correction reduced the raw multi-model ensemble annual rainfall bias from -307.13 mm (-26.9%) to -69.90 mm (-6.1%), representing an aggregate absolute bias reduction of 77.2% and an average model-specific bias reduction of 73.3% across the seven GCMs. Under near-term future warming (2021–2050), the multi-model ensemble projects modest increases in mean annual rainfall of +2.5% (+23.60 mm) under SSP2-4.5 and +1.2% (+11.70 mm) under SSP5-8.5. These findings provide an evidence-based foundation for flood hazard mitigation, reservoir operational scheduling, and climate adaptation in northern Thailand.

Keywords: CMIP6; ETCCDI; extreme precipitation; Quantile Delta Mapping; Uttaradit; Sirikit Dam; Northern Thailand.

# 1. Introduction

Precipitation extremes represent one of the most critical drivers of hydrologic vulnerability in Southeast Asia. In Northern Thailand, the province of Uttaradit occupies a pivotal hydrologic position within the Upper Nan River basin, hosting the Sirikit Multipurpose Dam...

# 2. Materials and Methods

Uttaradit Province covers approximately 7,838 km² in northern Thailand. Daily rainfall records were obtained from the Thai Meteorological Department (TMD) for 13 official rain gauge stations... Historical baseline was strictly locked to 1995–2014 per CMIP6 protocol.

# 3. Results

Baseline mean annual precipitation averaged 1139.80 mm. Seasonal partitioning demonstrates that 77.8% occurs during May–October. 11 ETCCDI indices confirm strong topographic contrasts across districts...

# 4. Discussion

The observed spatial variability reflects complex mountainous terrain flanking the Nan River basin. Pre-computed QDM bias correction substantially improves GCM fidelity...

# 5. Conclusions

This investigation delivers an authoritative, reproducible baseline and climate projection assessment for Uttaradit Province across 13 stations...

Table 1. Geographic and climatological characteristics of the 13 meteorological stations in Uttaradit Province (1995–2014 baseline).

| Station ID | District | Latitude (°N) | Longitude (°E) | Elevation (m) | Mean Ann (mm) | SD (mm) | CV (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 351001 | Phichai | 17.23 | 100.10 | 54.57 | 995.65 | 125.39 | 12.6 |
| 351002 | Tron | 17.42 | 100.13 | 62.02 | 1053.80 | 264.84 | 25.1 |
| 351003 | Tha Pla | 17.67 | 100.68 | 371.77 | 1425.06 | 204.80 | 14.4 |
| 351004 | Laplae | 17.65 | 100.38 | 115.76 | 1085.21 | 240.06 | 22.1 |
| 351005 | Mueang Uttaradit (West) | 17.60 | 100.05 | 81.47 | 1121.00 | 246.75 | 22.0 |
| 351006 | Nam Pat | 17.92 | 100.88 | 348.79 | 1129.59 | 293.55 | 26.0 |
| 351007 | Fak Tha | 18.02 | 101.07 | 427.85 | 1221.05 | 190.82 | 15.6 |
| 351008 | Thong Saen Khan | 17.47 | 100.37 | 82.43 | 1146.01 | 201.99 | 17.6 |
| 351009 | Laplae (North) | 17.75 | 100.28 | 95.27 | 1223.36 | 119.60 | 9.8 |
| 351010 | Nam Pat (Valley) | 17.90 | 100.82 | 214.69 | 1204.22 | 116.49 | 9.7 |
| 351011 | Mueang Uttaradit (South) | 17.56 | 100.11 | 65.37 | 1159.21 | 264.73 | 22.8 |
| 351012 | Ban Khok | 17.75 | 100.92 | 415.07 | 947.71 | 242.69 | 25.6 |
| 351201 | Uttaradit (Agromet/Synoptic) | 17.62 | 100.10 | 67.05 | 1105.59 | 259.72 | 23.5 |